MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 8/3/98											BLIZZARD

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-950

TRADE NAME: BLIZZARD

SYNONYMS: Silicone Coating

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 2, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

None				NA		NA			NA			NA			NA

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 350-500 °F

pH: NA

VAPOR PRESSURE: 0.03 mm Hg @ 20 °C

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): 100

SPECIFIC GRAVITY OR BULK DENSITY: 0.8-0.90

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE: Light red liquid

ODOR: Petroleum odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 105 °F (TCC)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Dry chemical, carbon dioxide, water and fog

SPECIAL FIRE FIGHTING PROCEDURES: Do not inhale smoke, fumes or vapors. A straight stream of water may spread the fire. However, water sprayed from as far away as possible may cool container.

UNUSUAL FIRE & EXPLOSION HAZARDS: Vapors are heavier than air and may travel a considerable distance to ignition and flashback. Combustible liquid: Keep away from ignition sources.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Ignition sources, hot glowing surfaces, sparks and open flame

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents, strong acids

HAZARDOUS DECOMPOSITION: Carbon monoxide, hydrogen chloride

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Eyes: Irritant. Skin: Irritant. Inhalation: Spray mist is irritating to the respiratory tract. Ingestion: Harmful or fatal if swallowed.

CHRONIC EFFECTS OF OVER EXPOSURE: NA

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None

VENTILATION: Local exhaust: Fresh air		Mechanical: Not recommended

PROTECTIVE CLOTHING: To avoid wetting skin and clothes

EYE PROTECTION: If splashing is to occur, wear safety glasses.

PROTECTIVE GLOVES: If skin contact is to be made, wear rubber gloves.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all sources of ignition; ventilate area. Wear appropriate clothing. Collect the spill with an absorbent. Do not allow spill to enter sewers or waterways. Product is toxic to fish and wildlife.

WASTE DISPOSAL METHOD: According to all applicable environmental and pollution standards

PRECAUTIONS IN HANDLING: Keep container closed when not in use. Follow all label directions. Store in a cool, dry place away from any ignition source. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.