MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 8/7/98											ADD ON

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: CRC-955

TRADE NAME: ADD ON

SYNONYMS: Ice Melter Additive

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 0, Reactivity: 0, Special Hazard: X

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Calcium Chloride			10043-52-4	96			15 mg/m3			15 mg/m3			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: ~6.00-9.00

VAPOR PRESSURE: <0.010 mm Hg @ 68°F

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: NA

SOLUBILITY IN WATER: Soluble

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Powder

ODOR: No odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Media appropriate for surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Not considered a fire hazard.

UNUSUAL FIRE & EXPLOSION HAZARDS: None known

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Metals/metal blends

HAZARDOUS DECOMPOSITION: NA

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin

HEALTH HAZARDS: Eyes: Contact may cause irritation, burns and eye damage. Skin: Contact may cause irritation and burns. Inhalation: May cause irritation. Ingestion: May cause irritation and tissue ulceration.

CHRONIC EFFECTS OF OVER EXPOSURE: None known

MEDICAL CONDITIONS AGGRAVATED: Disorders of the skin

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention immediately.

INGESTION: Induce vomiting immediately by giving two glasses of water and sticking finger down throat. Never give anything by mouth to an unconscious person. Seek medical attention immediately.

INHALATION: Move to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Call a physician.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If exposure limits are exceeded, use a NIOSH/MSHA respirator approved for your conditions of exposure.

VENTILATION: Adequate ventilation is required to minimize exposure or to maintain exposure levels below OSHA/ACGIH requirements.

PROTECTIVE CLOTHING: Clean body covering clothing

EYE PROTECTION: Safety glasses or chemical goggles

PROTECTIVE GLOVES: Impervious gloves

OTHER PRECAUTIONS: Safety shower, eyewash station and washing facilities should be readily available.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Sweep, shovel or pick up and return to dry, clean container. Do not use water unless more than 100 volumes of water are available for each volume of liquid ammonia.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Keep container closed when not in use. Store in a cool, dry place. Keep out of reach of children. Can cause corrosion of iron and steel at elevated temperatures.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.