MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 6/9/98												CHIP

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-160

TRADE NAME: CHIP

SYNONYMS: Chewing Gum Remover and Candle Wax Remover

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1 , Fire: 4, Reactivity: 0, Personal Protection: 3

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Isobutane/Propane Blend		75-28-5		NA			800 ppm			800 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: PSIG @ 70°F (Aerosols): 70

VAPOR DENSITY: NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY(H2O=1): Concentrate Only: 0.5379

SOLUBILITY IN WATER: Slight

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Clear spray

ODOR: Odorless

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: Extremely flammable

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): LEL: 2.0     UEL: 10.0

EXTINGUISHING MEDIA: Foam, dry chemical, carbon dioxide, water

SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus

UNUSUAL FIRE & EXPLOSION HAZARDS:  Do not expose aerosols to temperatures above 130°F or the container may rupture.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Open flame, welding arcs, heat

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents

HAZARDOUS DECOMPOSITION: CO, CO2

HAZARDOUS POLYMERIZATION:  Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation

HEALTH HAZARDS: Inhalation: Excessive inhalation of vapors can be harmful and may cause headache, dizziness, asphyxia, anesthetic effects and possible unconsciousness. Vapors are heavier than air and displace oxygen required for breathing. Eyes: May cause burning. Skin: May cause burns and frostbite.

CHRONIC EFFECTS OF OVER EXPOSURE: Unknown

MEDICAL CONDITIONS AGGRAVATED: May aggravate existing eye, skin or upper respiratory conditions.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritated, seek medical attention. If frostbite, warm skin slowly.

EYES: Flush with water for 15 minutes. If irritated, seek medical attention.

INGESTION: Unlikely route of exposure.

INHALATION: Remove to fresh air. Resuscitate if necessary. Get medical attention. Give oxygen.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  If vapor concentration exceeds TLV, use respirator approved by NIOSH to be used in a positive pressure mode.

VENTILATION: Adequate ventilation to keep vapor concentration below TLV.

PROTECTIVE CLOTHING: None

EYE PROTECTION: Safety glasses recommended

PROTECTIVE GLOVES: Rubber gloves recommended

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Wash hands thoroughly before handling food. Remove contaminated clothing.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Ventilate area to disperse vapors. DO NOT FLUSH TO SEWER. Remove ignition sources.

WASTE DISPOSAL METHOD: Aerosol cans, when vented to atmospheric pressure through normal use, pose no disposal hazard.

PRECAUTIONS IN HANDLING: Do not puncture or incinerate containers. Do not store at temperatures above 130°F. Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.