MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 1/19/99										VANISH

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-161

TRADE NAME: VANISH

SYNONYMS: Carpet Spotter

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 3, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Isopropyl Alcohol			67-63-0		10-20			400 ppm			400 ppm			No

Dipropylene Glycol Methyl Ether	34590-94-8	5-10			100 ppm			100 ppm			No

Liquefied Petroleum Gas		68476-85-7	5-10			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~198°F

VAPOR PRESSURE: 50 mm Hg

VAPOR DENSITY: > 1

SPECIFIC GRAVITY OR BULK DENSITY: 0.967

PERCENT VOLATILE BY WEIGHT (%): ~32

SOLUBILITY IN WATER:  Complete

APPEARANCE and ODOR:  Cloudy liquid, slight ammonia odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT (of Concentrate Only): 37°F (TOC)

FLAMMABILITY LIMITS IN AIR (% V): Non-flammable spray

EXTINGUISHING MEDIA: CO2, foam, dry media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire exposed container to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: At elevated temperatures (above 120°F) container may vent, rupture or burst.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY:  Stable

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon monoxide, carbon dioxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: May cause slight irritation but does not injure eye tissue. Skin: Frequent or prolonged contact may cause irritation. Inhalation: Mist can cause irritation of nasal and respiratory passages. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting, and diarrhea. Aspiration of material into the lungs can cause chemical pneumonitis. Minimal toxicity.

CHRONIC EFFECTS OF OVER EXPOSURE: Prolonged use of product may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash exposed skin with soap and water . If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get medical attention immediately.

INGESTION: DO NOT induce vomiting. Get medical attention immediately.

INHALATION: Remove to fresh air. Seek medical attention immediately.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section 2 ingredients below acceptable limits.

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions. Use chemical resistant gloves if hand contact will be made.

WORK/HYGIENIC PRACTICES: Don't eat, drink or smoke in work area. Wash hands before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked up absorbent material in accordance with Federal, State and local laws.

WASTE DISPOSAL METHOD: Aerosol cans, when emptied and depressurized through normal use, pose no disposal hazard and should be recycled. Consult Federal, State and local authorities for approved procedures.

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN. For Industrial and Institutional Use Only. Store in a cool, dry area away from heat or open flame. Do not store at temperatures above 120°F. NFPA Code 30B Rating: Level 1 Aerosol.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.