MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 10/23/01							GUARDIAN

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER:  RCC-224

TRADE NAME:  Guardian

SYNONYMS:  Protectant, Conditioner & Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1 , Fire: 0 , Reactivity: 0; Personal Protective Equipment: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Food Grade Mineral Oil		8042-47-5		10-20			5mg/m3			5mg/m3			NO

Aliphatic Mineral Spirits		64741-65-7	80-100			100 ppm			100 ppm			NO

Liquefied Petroleum Gas		68476-85-7	05-10			1000 ppm			1000 ppm			NO

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~200oF

pH: ~8.2

VAPOR PRESSURE OF CAN (psig @ 70oF): 55

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~ 15

SPECIFIC GRAVITY OR BULK DENSITY: 0.937

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Thick White Mousse

ODOR: Leather

TOTAL VOC %: ~15%

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None to Boiling (~212oF - T.O.C.)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): Non-Flammable Spray

EXTINGUISHING MEDIA: Foam, CO2, Dry Media

SPECIAL FIRE FIGHTING PROCEDURES: Cool fire exposed containers to prevent rupturing.  Fire Fighters should wear self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperature above 120oF may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CONDITIONS TO AVOID: NA

CHEMICAL INCOMPATIBILITY: Avoid contract with strong oxidizing agents.  Do not mix with other chemicals.

HAZARDOUS DECOMPOSITION: Carbon Dioxide, Carbon Monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes/Skin: can cause irritation. Inhalation: can cause irritation of nasal & respiratory passages. Ingestion: can cause nausea, vomiting, and 			   diarrhea.

CHRONIC EFFECTS OF OVER EXPOSURE: Irritation to upper respiratory tract, dizziness, nausea and other central nervous system effects.

MEDICAL CONDITIONS AGGRAVATED: Aspiration of material into lungs can cause chemical pneumonitis.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. If irritation persists, seek medical attention immediately.

INGESTION: Do not induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air.  If breathing stops give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed if used in accordance with label directions.

VENTILATION: Local exhaust to keep TLV of Section II ingredients below acceptable limits.

PROTECTIVE CLOTHING: None needed

EYE PROTECTION: Chemical resistant goggles if splash or spray back is anticipated.

PROTECTIVE GLOVES: Chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: None

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate. Maintain local exhaust & adequate ventilation. No smoking. Cover with absorbent material & sweep 				     up. Wash area to prevent slipping.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in a cool, dry place. Keep out of reach of children. Read all instructions on label before using.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.