MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: January 12, 2000									STA-PUT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-233

TRADE NAME: STA-PUT

SYNONYMS: Pressurized gelled citrus based cleaner

HMIS RATING (0: Insignificant; 1: Slight; 2: Moderate; 3: High; 4: Extreme): Health: 1, Reactivity: 0, Flammability: 2

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)	CAS NUMBER	APPROX. % WT	OSHA PEL	ACGIH TLV	EPA 40 CFR: 302	355	372

Mineral Spirits		64742-48-8	20.0-30.0		N/E		N/E			      N	N	N

Ethylene Glycol		107-21-1		<1.00		50ppm		50ppm			      Y	N	Y

Carbon Dioxide		124-38-9		1.00-10.0		10,000ppm	5,000ppm			      N	N	N

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT(°F): N/D

pH: N/A

VAPOR PRESSURE: N/D

VAPOR DENSITY (Air=1): >N/D

SPECIFIC GRAVITY OR BULK DENSITY (Water=1): 0.84

VOC CONTENT (% by weight): 85.0-95.0

SOLUBILITY IN WATER: Emulsifies

EVAPORATION RATE (WATER =1): >1.0

APPEARANCE and ODOR: Clear, orange, viscous liquid; citrus odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 122°F (concentrate)	(TEST METHOD): Closed cup	FLAME EXTENSION: N/D	NFPA 30B Rating: 3

FLAMMABILITY LIMITS IN AIR (% V): 	UPPER: N/E	LOWER: N/E

EXTINGUISHING MEDIA: Water spray

SPECIAL FIRE FIGHTING PROCEDURES: Cool fire-exposed containers with water fog. Firefighters should be equipped with full protective gear including self-contained breathing apparatus.

UNUSUAL FIRE & EXPLOSION HAZARDS: Contents under pressure. Exposure to temperatures above 120°F may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Extreme temperatures

CHEMICAL INCOMPATIBILITY: Strong acids and oxidizers

HAZARDOUS DECOMPOSITION: Thermal decomposition may produce oxides of carbon

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Ingestion, Eye Contact, Skin Contact

HEALTH HAZARDS: Eyes: Moderate irritation and redness; Skin: irritation with prolonged exposure, defatting; Swallowing: gastrointestinal irritation, nausea, cramps, vomiting; Inhaled: irritation to upper respiratory tract

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with water for 15 minutes

EYES: Flush under eyes and eyelids with plenty of cool water for at least 15 minutes. If irritation persists, obtain medical attention.

INGESTION: Contact physician or poison control center immediately. Do not induce vomiting.

INHALATION: Remove person to fresh air.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use with adequate ventilation. If recommended Exposure Limits are exceeded, wear a NIOSH approved respirator, following manufacturer’s recommendations.

VENTILATION: 	LOCAL: Recommended	MECHANICAL: Not required

EYE PROTECTION: Safety glasses or goggles

PROTECTIVE GLOVES: Chemical resistant

OTHER PROTECTIVE CLOTHING: None

OTHER PRECAUTIONS: Keep out of reach of children. Do not puncture or incinerate container.

PRECAUTIONS IN HANDLING AND STORAGE: Store in a cool dry place away from heat or open flame.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Ventilate area and remove all sources of ignition. Soak up material with inert absorbent material and place in a labeled closed container for disposal.

WASTE DISPOSAL METHOD: Consult local environmental authorities. Dispose of cans in non-incinerated trash only.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately	   N/D = Not Determined

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.