MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: June 6, 2000									MYSTIFY

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-241

TRADE NAME: MYSTIFY

SYNONYMS: Industrial Foam Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 0, Reactivity: 0 Protective Equipment: *A (HMIG)

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

2 Butoxy Ethanol			111-76-2		4.85			50 ppm			50 ppm			Yes

Sodium Metasilicate			6834-92-0		01-05			NE			NE			Yes

Liquefied Petroleum Gas		68476-85-7	1.0-5.0			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT: 212°F

pH: 12.8

VAPOR PRESSURE (psig @70°F): 55

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY (H2o=1): 1.027

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE and ODOR: Foaming spray, opaque/lemon odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT(of Concentrate Only): None to Boiling (212°F)

FLAMMABILITY LIMITS IN AIR (% V):  Non-flammable Spray

EXTINGUISHING MEDIA: Foam, CO2, Dry Media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperature above 120°F may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: Carbon monoxide, carbon dioxide

HAZARDOUS POLYMERIZATION: Will not occur.

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, eyes, skin and ingestion.

SIGNS AND SYMPTOMS OF EXPOSURE: Eyes: may cause slight irritation but does not injure eye tissue; Skin: Frequent or prolonged contact may cause irritation; Inhalation: Inhalation of mist can cause irritation of nasal and respiratory passages. Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects; Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea. Aspiration of material into the lungs can cause chemical pneumonitis. Minimal toxicity.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get medical attention immediately.

INGESTION: Do not induce vomiting. Seek medical attention immediately.

INHALATION: Remove to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions. Use chemical resistant gloves if hand contact will be made.

WORK/HYGIENIC PRACTICES: Ensure strict sanitary conditions are conformed to when working around chemicals.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow foam to dissipate and all propellant to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked absorbent materials in accordance with Federal, State, and local laws.

WASTE DISPOSAL METHOD: Aerosol cans, when emptied and depressurized through normal use, pose no disposal hazard and should be recycled. Consult Federal, State, and local authorities for approved procedures.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately	NE = Not Established

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.