MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 1/19/99							TROUBLE SHOOTER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-242

TRADE NAME: TROUBLE SHOOTER

SYNONYMS: Multi-Surface Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 4, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Toluene				108-88-3		40-60			100 ppm			100 ppm			Yes

Isopropyl Alcohol			67-63-0		5-10			400 ppm			400 ppm			No

Diacetone Alcohol			123-42-2		5-10			50 ppm			50 ppm			No

Liquified Petroleum Gas		68476-85-7	10-20			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~194°F

VAPOR PRESSURE of can (psig @ 70°F): 45

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~87

SPECIFIC GRAVITY OR BULK DENSITY: 0.913

SOLUBILITY IN WATER: Emulsifies

APPEARANCE and ODOR: Creamy white jel, mild odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT (of concentrate only): 54°F (T.O.C.)

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, dry chemical, carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120°F may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon monoxides, carbon dioxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin

HEALTH HAZARDS: Eyes: May cause slight irritation but does not injure eye tissue. Skin: Frequent or prolonged contact may cause irritation. Inhalation: Can cause irritation of nasal and respiratory passages. Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects. Ingestion: Can cause gastrointestinal irritation, nausea and vomiting.

CHRONIC EFFECTS OF OVER EXPOSURE: See Health Hazards.

MEDICAL CONDITIONS AGGRAVATED: Not known

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with cool water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section II Ingredients below acceptable limits.

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions. Use chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate. maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping. dispose of soaked absorbent material in accordance with Federal, State and Local laws.

WASTE DISPOSAL METHOD: Aerosol cans, when emptied and depressurized through normal use, pose no disposal hazard and should be recycled. Consult Federal, State and Local authorities for approved procedures.

PRECAUTIONS IN HANDLING: Do not store at temperatures above 120°F. Do not puncture or incinerate containers. Keep out of reach of children. Store in accordance with NFPA 30B for Level 2 Aerosols. For industrial and institutional use only. Store in a cool, dry area away from heat or open flame. Read label precautions carefully.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.