Latest Revision Date: 4/16/98											SPARKLE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-245

TRADE NAME: SPARKLE

SYNONYMS: Glass Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 2, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

2 Butoxy Ethanol			111-76-2		3.9			25 ppm			25 ppm			Yes

Ethyl Alcohol			64-17-5		10-20			1000 ppm			1000 ppm			No

Methyl Alcohol			67-56-1		0.8			200 ppm			200 ppm			Yes

Liquified Petroleum Gas		68476-85-7	1-5			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

VAPOR PRESSURE: 140 @ 130°F

VAPOR DENSITY: >1

SPECIFIC GRAVITY OR BULK DENSITY: <1

SOLUBILITY IN WATER:  Complete

EVAPORATION RATE (BuAc = 1): <1

APPEARANCE:  Wide hollow cone spray, semi-stable foam

ODOR: Solvent/floral odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  Non-flammable

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): LEL: 1.8 	UEL: 9.2

EXTINGUISHING MEDIA: Carbon dioxide, foam and/or dry chemical may be used.

SPECIAL FIRE FIGHTING PROCEDURES: Containers should be cooled with water to prevent vapor pressure build up. Use equipment or shielding, as required, to protect personnel from bursting, rupturing or venting containers.

UNUSUAL FIRE & EXPLOSION HAZARDS:  At elevated temperatures (over 54°C-130°F) containers exposed to direct flame or heat contact should be cooled with water to prevent weakening of container structure.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY:  Stable

Conditions To Avoid: Do not store above 54°C - 130°F. Keep away from heat, direct sunlight, open flames or sparks. Dropping of containers may cause bursting.

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents, heat and open flame

HAZARDOUS DECOMPOSITION: Carbon monoxide, carbon dioxide and additional toxic chemicals may be formed in small amounts.

HAZARDOUS POLYMERIZATION: NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Eyes, Skin, Ingestion

HEALTH HAZARDS: Respiratory irritation. High concentrations may cause dizziness and nausea. Possible skin absorption. Eye contact may cause burning and irritation.

CHRONIC EFFECTS OF OVER EXPOSURE: Prolonged or repeated skin contact may lead to drying, irritation and dermatitis.

MEDICAL CONDITIONS AGGRAVATED: Pre-existing skin or eye disorders may be aggravated by this product.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Remove contaminated clothing. Flush skin with water, follow by washing with soap and water. If irritation occurs, get medical attention. Do not reuse clothing until cleaned.

EYES: Flush eyes with plenty of water for 15 minutes while holding eyelids open. Get prompt medical attention.

INGESTION: Contact local poison control center or physician immediately.

INHALATION: Remove victim to fresh air and provide oxygen if breathing is difficult. Give artificial respiration if not breathing.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None is required if good ventilation is maintained. If exposure exceeds occupational exposure limits, use a NIOSH approved respirator to prevent over exposure.

VENTILATION: Local exhaust is adequate under normal conditions; mechanical ventilation is optional.

PROTECTIVE CLOTHING: Wear impervious clothing to prevent skin contact.

EYE PROTECTION: Chemical safety glasses are recommended.

PROTECTIVE GLOVES: Chemical resistant gloves

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Ensure strict sanitary conditions are conformed to when working around chemicals.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Remove all sources of ignition and ventilate area. Soak up spill with an inert absorbent and place into a designated disposal container.

WASTE DISPOSAL METHOD: Do not puncture or incinerate containers. When contents are depleted continue to depress button until all gas is expelled. Dispose of container in accordance with Federal, State and Local regulations.

PRECAUTIONS IN HANDLING: Avoid breathing vapor. Keep away from heat and flame. Use with adequate ventilation. Do not puncture or incinerate containers. Do not expose to direct sunlight  or store at temperatures above 130°F (54°C). Please read and follow the directions on the product label; they are  your best guide to using this product in the most effective way.

NA = Not Applicable     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.