Latest Revision Date: April 29, 2000								STICK-UM

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-250

TRADE NAME: STICK-UM

SYNONYMS: Heavy Duty Adhesive Spray

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme):  Health: 2, Flammability: 4, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV	OTHER LIMITS

Propane				74-98-6		NA			1000 ppm			NE		NA

Isobutane				106-97-8		NA			800 ppm			800 ppm		NA

Acetone (1)			67-64-1		NA			750 ppm			750 ppm		1000 ppm

Hexane				110-54-3		NA			50 ppm			50 ppm		NA

(1) WARNING: This product contains a chemical(s) that are known to the state of California to cause Cancer and reproductive harm. Section 313 Supplier Notification - Indicates hazardous ingredients which are toxic chemicals subject to the reporting requirements of Section 313 of the Emergency Planning and Community Right-To-Know Act of 1986 and of 40 CFR 372.

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NE

pH:  NA

VAPOR PRESSURE: PSIG @ 70°F (Aerosols): Max 80

VAPOR DENSITY: NE

PERCENT VOLATILE BY WEIGHT (%): NE

SPECIFIC GRAVITY (H2O = 1): Concentrate Only: 0.72

SOLUBILITY IN WATER:  Insoluble

EVAPORATION RATE (BuAc = 1): Faster

APPEARANCE:  Clear white liquid

ODOR: Solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  Extremely flammable

AUTOIGNITION TEMPERATURE: NE

FLAMMABILITY LIMITS IN AIR (% V): NE

EXTINGUISHING MEDIA: Dry chemical

SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus. Use water fog to cool container to prevent rupturing and exploding containers. Provide shielding for personnel.

UNUSUAL FIRE & EXPLOSION HAZARDS: Do not expose aerosols to temperatures above 130°F or the containers may rupture, vent, or burst.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CHEMICAL INCOMPATIBILITY: Strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin

HEALTH HAZARDS: Inhalation: Excessive inhalation of vapors can cause nasal and respiratory irritation, dizziness, weakness, nausea, headache, possible unconsciousness or asphyxiation. Eyes: Irritation. Skin: Irritation due to defatting of skin. Ingestion: Possible chemical pneumonitis if aspirated into lungs.

CHRONIC EFFECTS OF OVER EXPOSURE: Excessive inhalation of hexane may cause nerve damage.

MEDICAL CONDITIONS AGGRAVATED: May aggravate existing eye, skin or upper respiratory conditions.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritated, seek medical attention.

EYES: Flush with water for 15 minutes. Seek medical attention.

INGESTION: DO NOT INDUCE VOMITING. Get immediate medical attention.

INHALATION:  Remove to fresh air. Resuscitate if necessary. Get medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  None required for normal use.

VENTILATION: Adequate ventilation to keep vapor concentration below TLV.

PROTECTIVE CLOTHING: None required.

EYE PROTECTION: Safety glasses or goggles.

PROTECTIVE GLOVES: Wear rubber gloves.

OTHER PRECAUTIONS: Read label precautions carefully.

WORK/HYGIENIC PRACTICES: Wash with soap and water before handling food. Remove contaminated clothing.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Absorb with suitable medium. Incinerate or landfill according to Federal, State and Local regulations. DO NOT FLUSH TO SEWER.

WASTE DISPOSAL METHOD: Aerosol cans when vented to atmospheric pressure through normal use pose no disposal hazard.

PRECAUTIONS IN HANDLING: Do not puncture or incinerate containers. Do not store at temperatures above 130°F. KEEP OUT OF REACH OF CHILDREN. Avoid food contamination. Avoid breathing vapors. Remove ignition sources.

NA = Not Available     NE = Not Established	<= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.