MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 6/13/01									BLASTER

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-254

TRADE NAME: BLASTER

SYNONYMS: Lint & Dust Remover

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 4, Reactivity: 1, Protective Equipment: A

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

LIQUIFIED GAS MIXTURE				70-80									NO

Dimethyl ether			115-10-6		NE			NE			NE			NO

Isobutane				75-28-5		NE			NE			NE			NO

Propane				74-98-6		NE			1000 ppm			NE			NO

1, 1, 1, 2 Tetrafluoroethane		811-97-2		20-30			NE			NE			NO

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

VAPOR PRESSURE (psig): 158@130F

VAPOR DENSITY (Air=1): >1

SPECIFIC GRAVITY OR BULK DENSITY: <1

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (Ether = 1): <1

APPEARANCE: Clear, colorless

ODOR: Ethereal odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT(0F Propellant): NA (Concentrate)

FLAMMABILITY LIMITS IN AIR (% V)(Propellant): (approx)LEL: 1.8		UEL: 18.0

EXTINGUISHING MEDIA: Carbon dioxide, foam and/or dry chemical may be used.

SPECIAL FIRE FIGHTING PROCEDURES: Containers should be cooled with water to prevent vapor pressure build up. Use equipment or shielding, as required, to protect personnel from bursting, rupturing or venting containers.

UNUSUAL FIRE & EXPLOSION HAZARDS: At elevated temperatures (over 54°C-130°F) containers exposed to direct flame or heat contact should be cooled with water to prevent weakening of container structure.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CONDITIONS TO AVOID: Do not store above 54°C-130°F. Keep away from heat, direct sunlight, open flames or sparks. Dropping of containers may cause bursting.

CHEMICAL INCOMPATIBILITY: Oxidizing agents

HAZARDOUS DECOMPOSITION PRODUCTS: Hydroflouric acid, carbonyl fluoride, oxides of carbon and additional toxic chemicals may be formed in small amounts.

HAZARDOUS POLYMERIZATION: NA

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Eyes, Skin, Ingestion

HEALTH HAZARDS: Inhalation of high concentrations of vapor is harmful and may cause heart irregularities, unconsciousness, or death. Eye or skin contact with liquid may cause freezing or frost bite. Ingestion is not considered a potential route of exposure.

MEDICAL CONDITIONS AGGRAVATED: Preexisting diseases of the central nervous system may be aggravated by exposure to this product.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

EYES: Flush eyes with large amounts of water and allow to evaporate.

INHALATION: If unconscious, remove victim to fresh air and call a physician.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None required if good ventilation is maintained. If exposure exceeds occupational exposure limits, use a NIOSH approved respirator to prevent overexposure.

VENTILATION: General ventilation is adequate under normal conditions, mechanical ventilation is optional.

EYE PROTECTION: Chemical safety glasses are recommended.

PROTECTIVE GLOVES: Chemical resistant gloves are recommended-consult glove manufacturer to determine the proper type for a specific operation.

OTHER PROTECTIVE CLOTHING: Wear impervious clothing to prevent skin contact.

WORK/HYGIENIC PRACTICES: Ensure strict sanitary conditions are conformed to when working around chemicals. Protective clothing and equipment should be in accordance with 29 CFR 1910.132 and CFR 1910.133.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: NA

WASTE DISPOSAL METHOD: NA

PRECAUTIONS IN HANDLING: NA

NA = Not Available     <= Less than     >= Greater than     ~= Approximately	NE = Not Established

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.