Latest Revision Date: 5/21/02							REVIVE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-262

TRADE NAME: Revive

SYNONYMS: Metal Cleaner, Polish & Protectant

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 2, Reactivity: 1, Personal Protection: B

SECTION: 2					INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Citrus Terpene			5989-27-5		10-20			400 ppm			400 ppm			No

Triethanolamine			102-71-6		01-05			N/E			N/E			No

Propylene Glycol Monomethyl Ether	34590-94-8	01-05			50 ppm			50 ppm			No

Stearyl Propionate			N/A		01-05			N/E			N/E			No

Liquefied Petroleum Gas		68476-85-7	05-10			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~176oF

pH: 8.6

VAPOR PRESSURE: 65

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~27%

SPECIFIC GRAVITY OR BULK DENSITY: 0.973

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): N/A

APPEARANCE: Milky White Foam

ODOR: Citrus Fragrance

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 130oF (T.O.C.)

AUTOIGNITION TEMPERATURE: N/A

FLAMMABILITY LIMITS IN AIR (% V): N/A

EXTINGUISHING MEDIA: Foam, CO2, Dry Media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing.  Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperature above 120oF may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Material Stable

Conditions To Avoid: N/A

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon Dioxide, Carbon Monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes:  May cause slight irritation but does not injure eye tissue.  Skin:  Frequent or prolonged contact may cause irritation.  Inhalation:  Inhalation of mist can cause irritation of nasal and respiratory passages.    Ingestion:  Can cause gastrointestinal irritation, nausea, vomiting and diarrhea.  Aspiration of material into the lungs can cause chemical pneumonitis.  Minimal toxicity.

CHRONIC EFFECTS OF OVER EXPOSURE: Skin:  Repeated exposure may result in excessive drying of the skin.  Inhalation:  Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects.

MEDICAL CONDITIONS AGGRAVATED: N/A

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with large amounts of water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get medical attention immediately.

INGESTION: Do not induce vomiting.  Seek medical attention immediately.

INHALATION: Remove to fresh air.  Seek medical attention immediately.  If breathing stops, give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

PROTECTIVE CLOTHING: None need for proper use in accordance with label directions.

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions.  Use chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: None

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate.  Maintain local exhaust and adequate ventilation.  Cover with absorbent material and sweep up.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Store in a cool, dry area away from heat or open flame.  Do not store at temperatures above 120oF.  Keep out of reach of children.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately     NE= Not Established