Latest Revision Date: 7/15/98											SURE-FOOT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-265

TRADE NAME: SURE-FOOT

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 4, Reactivity: 0, Personal Protection: I

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Acetone				67-64-1		20-40			1000 ppm			1000 ppm			No

Xylene				1330-20-7		10-20			150 ppm			100 ppm			Yes

Toluene				108-88-3		10-20			150 ppm			100 ppm			Yes

Titanium Dioxide			13463-67-7	0-10			NA			15 mg/ m3		No

Hydrocarbon Propellant		68478-85-7	20-40			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: -13 °F to 232 °F

VAPOR PRESSURE OF CAN (psig @ 70 °F): 65

VAPOR DENSITY:>1

PERCENT VOLATILE BY WEIGHT (%): ~77

SPECIFIC GRAVITY OR BULK DENSITY (H2O=1) @ 75 °F: 0.800

SOLUBILITY IN WATER: Spray mist is insoluble.

APPEARANCE: Clear liquid

ODOR: Solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: 45 °F

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, CO2, dry media, water fog

SPECIAL FIRE FIGHTING PROCEDURES: Hazardous decomposition products may form from incomplete combustion. Wear full protective clothing and self-contained positive pressure breathing apparatus. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Extremely flammable liquid and vapors. Containers can build pressure from heat and rupture explosively. Volatile vapors can burn in the open or explode if confined. Vapor is heavy and can travel long distances to source of ignition.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Strong oxidizing agents

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide and potentially other toxic gases if incomplete combustion occurs

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: Can cause severe irritation, redness, tearing and blurred vision. Skin: May cause irritation. Inhalation: Can cause irritation of nasal and respiratory passages. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea.

CHRONIC EFFECTS OF OVER EXPOSURE: Skin: Frequent or prolonged contact will cause irritation, defatting of the skin and dermatitis in some cases. May cause allergic skin reaction. Inhalation: Abusive or excessive inhalation can cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects. Repeated and prolonged exposure to solvent vapors may cause permanent brain and nervous system damage. May also cause liver and kidney damage. Ingestion: Aspiration of material into the lungs can cause chemical pneumonitis.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get medical attention immediately.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Remove to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration or administer oxygen.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If ventilation is not adequate to reduce vapors below TLV levels, use a NIOSH/MSHA approved carbon filter respirator for organic vapors.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

EYE PROTECTION: Wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: Use chemical resistant gloves if hand contact will be made.

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked absorbent material in accordance with Federal, State and Local laws.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities. Aerosol cans, when emptied and depressurized through normal use, pose no disposal hazard and should be recycled.

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN. For Industrial and Institutional use only. Store in a cool, dry area away from heat or open flame. Do not store at temperatures above 120 °F.

NA = Not Available     <= Less than     >= Greater than