Latest Revision Date: 05/28/02						ARMOUR COAT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-285

TRADE NAME: Armour Coat

SYNONYMS: Stainless Steel Coating

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 3, Reactivity: 1, Personal Protection: B

SECTION: 2					INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Parachlorobenzotrifluoride		98-56-6		20-40			N/E			N/E			NO

Isoparaffninic Hydrocarbon		64741-66-8	10-20			400ppm			400ppm			NO

Epoxy Ester Resin			Proprietary	05-10			100ppm			100ppm			NO

Stainless Steel (Alloy/Mixture)		N/A		01-05			N/E			N/E			NO

Liquefied Petroleum Gas		68476-85-7	20-40			1000ppm			1000ppm			NO

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~167oF

pH: N/A

VAPOR PRESSURE: 55

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~58%

SPECIFIC GRAVITY OR BULK DENSITY: 1.100

SOLUBILITY IN WATER: Negligible

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Silver Grey Liquid

ODOR: Solvent

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: ~44oF

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, CO2, Dry Media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing.  Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120oF may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Material stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon Dioxide, Carbon Monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: May cause slight irritation but does not injure eye tissue.  Ingestion:  Can cause gastrointestinal irritation, nausea, vomiting and diarrhea.  Aspiration of material into the lungs can cause chemical pneumonitis.  Inhalation: Inhalation of mist can cause irritation of nasal and respiratory passages.

CHRONIC EFFECTS OF OVER EXPOSURE: Skin: Frequent or prolonged contact may cause irritation.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN:  Wash with soap and water.  If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get  medical attention immediately.

INGESTION: Do not induce vomiting.  Seek medical attention immediately.

INHALATION: Remove to fresh air.  Seek medial attention immediately.  If breathing stops give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

PROTECTIVE CLOTHING: None needed for proper use in accordance with label directions.

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions.  Use chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate.  Maintain local exhaust and adequate ventilation.  Cover with absorbent material and sweep up.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: NA

NA = Not Available     <= Less than     >= Greater than     ~= Approximately     NE= Not Established