MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 8/12/98										BIG RED

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-560

TRADE NAME: BIG RED

SYNONYMS: Red Spray Grease

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 2, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Hexane				110-54-3		NA			50 ppm			50 ppm			Yes

Aromatic Petroleum Distillate		64742-94-5	NA			100 ppm			100 ppm			No

Isobutane				75-28-5		NA			800 ppm			800 ppm			No

Propane				74-98-6		NA			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: 46 mm Hg

VAPOR DENSITY (Air=1): 2.9

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 0.78

SOLUBILITY IN WATER: Nil

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE and ODOR: Slight viscous red liquid, sweetish solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, dry chemical, carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Use water fog to cool containers to prevent rupturing and bursting.

UNUSUAL FIRE & EXPLOSION HAZARDS: Do not expose aerosols to temperatures above 130°F or the container may rupture. Provide shielding to protect personnel.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Open flame, welding arcs, heat, sparks

CHEMICAL INCOMPATIBILITY: Oxygen, oxidants

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes

HEALTH HAZARDS: Inhalation: Can cause headaches, dizziness. Eyes: Irritation. Skin: can cause skin defatting. Ingestion: Possible chemical pneumonitis if aspirated into the lungs.

CHRONIC EFFECTS OF OVER EXPOSURE: None known

MEDICAL CONDITIONS AGGRAVATED: Asthma

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: DO NOT induce vomiting. Drink two large glasses of water. Seek medical attention immediately.

INHALATION: Move to fresh air. Resuscitate if necessary. Get medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Not normally needed

VENTILATION: Normal room ventilation

PROTECTIVE CLOTHING: None

EYE PROTECTION: Safety glasses

PROTECTIVE GLOVES: Solvent resistant

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Cover with absorbent material and sweep up. Wash area to prevent slipping.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Do not puncture or incinerate containers. So not store at temperatures above 130°F. KEEP OUT OF REACH OF CHILDREN.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.