MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/20/01								TIGHT SPOT

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-564

TRADE NAME: TIGHT SPOT

SYNONYMS: Penetrant, Lubricant, Demoisturant, Protectant

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 2, Reactivity: 0, Protective Equipment: G

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Trichloroethylene			79-01-6		60-80			50 ppm			50 ppm			Yes

Mineral Oil			8042-47-5		10-20			5 mg/M3			5 mg/M3			No

Ethylene Glycol Monobutyl Ether	111-76-2		01-05			25 ppm			25 ppm			Yes

Carbon Dioxide			124-38-9		01-05			10000 ppm		10000 ppm		No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg:  158° F

VAPOR PRESSURE: 90

VAPOR DENSITY: >1

SPECIFIC GRAVITY OR BULK DENSITY (H2O=1)@75°: 1.25

SOLUBILITY IN WATER:  Insoluble

TOTAL VOC %: ~87°

APPEARANCE and ODOR:  Transparent liquid, Solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT:  None

EXTINGUISHING MEDIA: Foam, dry media, CO2

SPECIAL FIRE FIGHTING PROCEDURES: Use self-contained breathing apparatus and protective clothing. Cool fire-exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS:  Exposure to temperatures above 120° F may cause bursting. Vapors concentrated in a confined or poorly ventilated area can be ignited upon contact with a high-energy spark, flame, or high-intensity source of heat. This can occur at concentrations ranging from 7.8-52%.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: Carbon Dioxide, Carbon Monoxide, Hydrogen Chloride & possible trace amounts of Phosgene.

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Eyes, Ingestion

HEALTH HAZARDS: Inhalation: Inhalation may result in nervous system depression. Inhalation of mist can cause irritation of nasal and respiratory passages. Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects including but not limited to ventricular fibrillation, cardiac failure or death. Eyes: Causes pain, redness and irritation. Skin: Frequent or prolonged contact may cause irritation. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea. Aspiration of material into the lungs can cause chemical pneumonitis.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with plenty of soap and water. If irritated, seek medical attention.

EYES: Flush with large amounts of cool running water for 15 minutes while holding upper and lower lids open. Seek medical attention immediately.

INGESTION: If conscious: Drink large amounts of water. Do not induce vomiting. Seek medical attention immediately. If unconscious: Do not attempt to give anything by mouth to an unconscious person. Seek medical attention immediately.

INHALATION: Remove to fresh air. If  continued difficulty is experienced, seek medical attention immediately. If breathing stops, give artificial respiration.

NOTES TO PHYSICIAN: Only administer Adrenaline after careful consideration following Trichloroethylene overexposure. Increased sensitivity of the heart to Adrenaline may be caused by overexposure to Trichloroethylene.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  None needed for proper use in accordance with label directions. If ventilation is not adequate to reduce vapors below Threshold Limit: Value (TLV) levels, use a NIOSH/MSHA approved air-purifying respirator equipped with an organic vapor cartridge.

VENTILATION: Provide local exhaust to keep TLV ingredients below acceptable limits.

EYE PROTECTION: Wear chemical proof splash goggles or face shield with safety glasses for splash protection.

PROTECTIVE GLOVES: Use chemical resistant gloves if hand contact will be made.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked absorbent material in accordance with Federal, State and local laws.

WASTE DISPOSAL METHOD: Aerosol cans when emptied and depressurized through normal use, pose no disposal hazard and should be recycled. Consult Federal, State, and local authorities for approved procedures.

NA = Not Available     <= Less than     >= Greater than

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.