MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 8/12/98										SLICK

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-566

TRADE NAME: SLICK

SYNONYMS: Penetrating Gel Lubricant With Teflon

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 3, Reactivity: 1, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Isoparaffinic Hydrocarbon		64741-66-8	10-20			400 ppm			400 ppm			No

Acetone				67-64-1		10-20			750 ppm			750 ppm			No

Liquefied Petroleum Gas		68476-85-7	40-60			1000 ppm			1000 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: 125°F

pH: NA

VAPOR PRESSURE: 50 mm Hg

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~78

SPECIFIC GRAVITY OR BULK DENSITY: 0.780

SOLUBILITY IN WATER: Moderate

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE and ODOR: Cloudy, tan liquid, solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: <10°F (TOC)

FLAMMABILITY LIMITS IN AIR (% V): Extremely flammable spray

EXTINGUISHING MEDIA: Foam, carbon dioxide, dry media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120°F may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: May cause irritation, pain, redness or burning. Inhalation: Mist can cause irritation of nasal and respiratory passages. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea.

CHRONIC EFFECTS OF OVER EXPOSURE: Skin: Frequent or prolonged contact may cause irritation. Inhalation: Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects. Aspiration of material into the lungs can cause chemical pneumonitis. Minimal toxicity.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritation persists.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

PROTECTIVE CLOTHING: NA

EYE PROTECTION: If splashing is to occur, wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: If hand contact will be made, wear chemical resistant gloves.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN. For industrial and institutional use only. Store in a cool, dry area away from heat or open flame. Do not store at temperatures above 120°F.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.