Latest Revision Date: 05/20/02						CLING

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-567

TRADE NAME: Cling

SYNONYMS: Foaming Chain and Cable Lubricant

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 1, Reactivity: 0, Personal Protection: B

SECTION: 2					INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

White Mineral Oil			8042-47-5		30-50			5 mg/m3(mist)		5 mg/m3(mist)		NO

Olefin Polymer			Proprietary	15-30			10mg/m3			5mg/m3			NO

Lithium Grease			N/A		05-10			N/E			N/E			NO

Oxidized Petroleum Hydrocarbon	68425-34-3

64742-52-5	05-10			100 ppm			100 ppm			NO

Dipropylene Glycol Butyl Ether	9003-27-4

64741-88-4	05-10			5mg/m3(mist)		5mg/m3(mist)		NO

Liquefied Petroleum Gas		68476-85-7	40-60			1000 ppm			1000 ppm			NO

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~114oF

pH: N/A

VAPOR PRESSURE: 45

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~40%

SPECIFIC GRAVITY OR BULK DENSITY: 0.865

SOLUBILITY IN WATER: Insoluble

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Thick Black Liquid

ODOR: Petroleum Odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: >222oF  (T.O.C.)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, CO2, Dry Media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing.  Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120oF may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Material stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon Dioxide, Carbon Monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: May cause slight irritation but does not injure eye tissue.  Ingestion:  Can cause gastrointestinal irritation, nausea, vomiting and diarrhea.  Aspiration of material into the lungs can cause chemical pneumonitis.  Minimal toxicity.  Inhalation:  Can cause irritation of nasal and respiratory passages.

CHRONIC EFFECTS OF OVER EXPOSURE: Skin:  Frequent or prolonged contact may cause irritation.  Inhalation:  May cause irritation to upper respiratory tract, dizziness, nausea and other central nervous system effects.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water.  If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get  medical attention immediately.

INGESTION: Do not induce vomiting.  Seek medical attention immediately.

INHALATION: Remove to fresh air.  Seek medial attention immediately.  If breathing stops give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

PROTECTIVE CLOTHING: NA

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions.  Use chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate.  Maintain local exhaust and adequate ventilation.  Cover with absorbent material and sweep up.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: No Smoking

NA = Not Available     <= Less than     >= Greater than     ~= Approximately     NE= Not Established