MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 05/14/01										SUPER SIL

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-605

TRADE NAME: SUPER SIL

SYNONYMS: 100% RTV Silicone Caulk

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 1, Fire: 1, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Silica, Amorphous			7631-86-9		05-10			10 ppm			10 ppm			NO

Ethyltriacetoxysilane		17689-77-9	01-05			10 ppm			10 ppm			NO

Methyltriacetoxysilane		4253-34-3		01-05			10 ppm			10 ppm			NO

Liquified Petroleum Gas		68476-85-7	01-05			1000 ppm			1000 ppm			NO

Acetic Acid is formed upon contact with water or humid air. Provide adequate ventilation to control exposures with OSHA guidelines.

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: NA

VAPOR DENSITY (Air=1): NA

SPECIFIC GRAVITY OR BULK DENSITY@77°F: 1.03

SOLUBILITY IN WATER: None

VOC %: NA

APPEARANCE: Clear or White

ODOR: Acetic Acid

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: NA

FLAMMABILITY LIMITS IN AIR (% V): Container pressurized with a Flammable Gas, as listed in Section II, Do Not remove rubber plug from bottom of container, or expose to heat, sparks or flames.

EXTINGUISHING MEDIA: Foam, CO2, Dry Media, Water

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire-exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120°F may cause burning.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Material stable

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: Thermal breakdown may result in production of Sulfur Dioxides, CO2, Silicon Dioxide, Formaldehyde and NO1.

HAZARDOUS POLYMERIZATION: Will not occur.

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Skin contact, Inhalation, Ingestion

HEALTH HAZARDS: Eyes: Direct contact may cause irritation associated with redness and swelling; Skin: Frequent or prolonged contact may cause irritation; Inhalation: Inhalation of mist can cause irritation of nasal and respiratory passages. Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea, and other central nervous system effects; Ingestion: Small amounts transferred to the mouth by fingers during use, etc., should not injure. Swallowing large amounts may cause digestive discomfort.

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wipe off excess sealant with dry paper towel or cloth and wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get medical attention immediately.

INGESTION: Do not induce vomiting. No additional first aid should be needed.

INHALATION: Remove to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: Use respiratory protection unless adequate local exhaust ventilation is provided.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

EYE PROTECTION: Wear safety glasses or chemical proof goggles. Use of product produces Acetic Acid vapors, as described in Section II, do not wear contact lens when using this product.

PROTECTIVE GLOVES: Use chemical resistant gloves if hand contact will be made.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Dispose of material in accordance with Federal, State, and local laws.

WASTE DISPOSAL METHOD: Consult Federal, State, and local authorities for approved procedures.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.