Latest Revision Date: 05/20/02						INVISIBLE GLOVE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-761

TRADE NAME: Invisible Glove

SYNONYMS: Protective Emollient Foam

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 0, Fire: 0, Reactivity: 0, Personal Protection: A

SECTION: 2					INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Stearic Acid			57-11-4		01-05			N/E			N/E			NO

Polydimethylsiloxane		63148-62-9	01-05			N/E			N/E			NO

Propylene Glycol			57-55-6		01-05			N/E			N/E			NO

Triethanolamine			102-71-6		01-05			N/E			N/E			NO

Isopropyl Myristate			110-27-0		01-05			N/E			N/E			NO

Liquefied Petroleum Gas		68476-85-7	05-10			1000 ppm			1000 ppm			NO

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~212oF

pH: 8.0

VAPOR PRESSURE: 64

VAPOR DENSITY (Air=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~5%

SPECIFIC GRAVITY OR BULK DENSITY: 1.00

SOLUBILITY IN WATER: Complete

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: White Foam

ODOR: Perfumed Scent

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None to Boiling (~212oF – T.O.C.)

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, CO2, Dry Media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing.  Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120oF may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Material stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents

HAZARDOUS DECOMPOSITION: Carbon Dioxide, Carbon Monoxide

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Eyes, Ingestion

HEALTH HAZARDS: Eyes: May cause slight irritation but does not injure eye tissue.  Ingestion:  Can cause gastrointestinal irritation, nausea, vomiting and diarrhea.  Aspiration of material into the lungs can cause chemical pneumonitis.  Minimal toxicity.

CHRONIC EFFECTS OF OVER EXPOSURE: NA

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: If, in rare cases, allergic reaction occurs, wash with soap and water.  If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get  medical attention immediately.

INGESTION: Do not induce vomiting.  Seek medical attention immediately.

INHALATION: Remove to fresh air.  Seek medial attention immediately.  If breathing stops give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: None

VENTILATION: None

PROTECTIVE CLOTHING: None

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate.  Maintain local exhaust and adequate ventilation.  Cover with absorbent material and sweep up.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: NA

NA = Not Available     <= Less than     >= Greater than     ~= Approximately     NE= Not Established