MATERIAL SAFETY DATA SHEET						COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053							1700 SOUTH FOURTH STREET

Information #: (402) 379-0100							NORFOLK, NE 68701

Latest Revision Date: 3/9/98											DITRON

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-825

TRADE NAME: DITRON

SYNONYMS: Solvent Degreaser

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health, 2, Fire, 0, Reactivity, 1, Personal Protection, G

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Trichloroethylene			79-01-6		80-100			50 ppm			50 ppm			Yes

Carbon Dioxide			124-38-9		1-5			10,000 ppm		10,000 ppm		No

Isopropyl alcohol			67-63-0		1-5			400 ppm			400 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~154.4°F

VAPOR PRESSURE: 90 mm Hg

VAPOR DENSITY: 4.53

PERCENT VOLATILE BY WEIGHT (%): 100

SPECIFIC GRAVITY OR BULK DENSITY: (@75°F) 1.419

SOLUBILITY IN WATER: Insoluble

APPEARANCE AND ODOR:  Clear, colorless liquid; mild, sweet odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None to boiling

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): Non-flammable spray

EXTINGUISHING MEDIA: Foam, dry chemical, carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120°F may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: NA

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide, hydrogen chloride and small amounts of phosgene

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin, Ingestion

HEALTH HAZARDS: Eyes and Skin: Contact with eyes and skin can cause severe irritation, redness, tearing and blurred vision.  Inhalation: Mist can cause irritation of nasal and respiratory passages.

CHRONIC EFFECTS OF OVER EXPOSURE: Prolonged exposure will cause defatting of the skin and dermatitis in some cases. It may cause allergic skin reaction. Repeated and prolonged exposure to high levels of solvent vapors may cause permanent brain and nervous system damage. May also cause liver and kidney damage. Excessive ingestion can cause gastrointestinal irritation, nausea, vomiting and diarrhea. Aspiration of material into lungs can cause chemical pneumonitis.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get medical attention immediately.

INGESTION: DO NOT induce vomiting. Seek medical attention.

INHALATION: Remove victim to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration or administer oxygen.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If ventilation is not adequate to reduce vapors below TLV levels, use a NIOSH/MSHA approved carbon filter respirator for organic vapors.

VENTILATION: Provide local ventilation, open windows and doors to ensure fresh air entry during application, to keep TLV of Section II ingredients below acceptable limits.

PROTECTIVE CLOTHING: None required

EYE PROTECTION: Wear approved safety glasses.

PROTECTIVE GLOVES: Use chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Wash hands after use.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. Cover with absorbent material and sweep up. Wash area to prevent slipping. Dispose of soaked absorbent material in accordance with Federal, State and Local laws.

WASTE DISPOSAL METHOD: Aerosol cans, when emptied and depressurized through normal use, pose no disposal hazard and should be recycled. Consult Federal, State and Local authorities for approved procedures.

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN. For industrial and institutional use only. Store in a cool, dry area away from heat or open flame. Do not store at temperatures above 120°F. NFPA CODE 30 B Rating: Level 1 Aerosol.

NA = Not Applicable     <= Less than     >= Greater than     ~= Approximately

THE INFORMATION CONTAINED IS BASED ON DATA CONSIDERED ACCURATE. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED AND FROM THE USE THEREOF.