MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 7/20/98									PURE SOLVE

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-826

TRADE NAME: PURE SOLVE

SYNONYMS: Aerosol Contact Cleaner

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 0, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

1,1-Dichloro-1-fluoroethane		1717-00-6		80-100			NA			NA			Yes

Carbon Dioxide Propellant		124-38-9		1-5			10,000 ppm		10,000 ppm		No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: ~90 °F

VAPOR PRESSURE OF CAN (psig @70 °F): 80

VAPOR DENSITY (AIR=1): >1

PERCENT VOLATILE BY WEIGHT (%): ~3

SPECIFIC GRAVITY OR BULK DENSITY (H2O=1) @ 75 °F: 1.250

SOLUBILITY IN WATER: Slight

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Clear, colorless liquid

ODOR: Ether-like odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: None to boiling

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): Non-flammable spray

EXTINGUISHING MEDIA: Foam, CO2, dry media

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus and protective clothing. Cool fire exposed containers to prevent rupturing.

UNUSUAL FIRE & EXPLOSION HAZARDS: Exposure to temperatures above 120 °F may cause bursting.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Strong oxidizing agents

CHEMICAL INCOMPATIBILITY: Avoid contact with strong oxidizing agents.

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide, hydrogen chloride, hydrogen fluoride and chlorine

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Ingestion

HEALTH HAZARDS: Eyes: May cause slight irritation but does not injure eye tissue. Skin: Frequent or prolonged contact may cause irritation. Inhalation: Inhalation of mist can cause irritation of nasal and respiratory passages. Ingestion: Can cause gastrointestinal irritation, nausea, vomiting and diarrhea.

CHRONIC EFFECTS OF OVER EXPOSURE: Inhalation: Abusive or excessive inhalation may cause irritation to the upper respiratory tract, dizziness, nausea and other central nervous system effects. Ingestion: Aspiration of material into the lungs can cause chemical pneumonitis.

MEDICAL CONDITIONS AGGRAVATED: NA

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek medical attention.

EYES: Flush with large amounts of cool running water for at least 15 minutes while holding upper and lower lids open. If irritation persists, get medical attention immediately.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Remove to fresh air. Seek medical attention immediately. If breathing stops, give artificial respiration.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:  None needed for proper use in accordance with label directions.

VENTILATION: Provide local exhaust to keep TLV of Section II ingredients below acceptable limits.

PROTECTIVE CLOTHING: NA

EYE PROTECTION: None needed unless it is anticipated that a splash or spray back will occur, then wear safety glasses or chemical proof goggles.

PROTECTIVE GLOVES: None needed for proper use in accordance with label directions. Use chemical resistant gloves if hand contact will be made.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Allow propellant and solvent to evaporate. Maintain local exhaust and adequate ventilation. No smoking. Keep sparks, heat sources and open flame far away from spill or leak. No additional cleaning should be required.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities. Aerosol cans, when emptied and depressurized through normal use, pose no disposal hazard and should be recycled.

PRECAUTIONS IN HANDLING: KEEP OUT OF REACH OF CHILDREN.  For industrial and institutional use only. Store in a cool, dry area away from heat or open flame. Do not store at temperatures above 120 °F.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately