MATERIAL SAFETY DATA SHEET			COLONIAL RESEARCH CHEMICAL CORP.

Emergency #: 1-800-535-5053				1700 SOUTH FOURTH STREET

Information #: (402) 379-0100				NORFOLK, NE 68701

Latest Revision Date: 8/13/98										GUT-LESS

SECTION: 1					PRODUCT IDENTIFICATION

PRODUCT NAME OR NUMBER: RCC-827

TRADE NAME: GUT-LESS

SYNONYMS: Paint, Varnish, Decal and Gasket Remover

HMIS RATING (0: Minimal: 1: Slight; 2: Moderate: 3: Serious; 4: Extreme): Health: 2, Fire: 1, Reactivity: 0, Personal Protection: B

SECTION: 2					HAZARDOUS INGREDIENTS/HAZARD DATA

CHEMICAL NAME(S)		CAS NUMBER	APPROX. % WT		OSHA PEL		ACGIH TLV		SEC 313

Methylene Chloride			75-09-2		NA			100 ppm			50 ppm			Yes

Petroleum Distillates		8052-41-3		NA			500 ppm			100 ppm			No

Methanol				67-56-1		NA			200 ppm			200 ppm			Yes

Propane				74-98-6		NA			1000 ppm			1000 ppm			No

Isobutane				75-28-5		NA			800 ppm			800 ppm			No

SECTION: 3					PHYSICAL DATA

BOILING/MELTING POINT @ 760mmHg: NA

pH: NA

VAPOR PRESSURE: Max. 60

VAPOR DENSITY (Air=1): NA

PERCENT VOLATILE BY WEIGHT (%): NA

SPECIFIC GRAVITY OR BULK DENSITY: 1.250

SOLUBILITY IN WATER: Partial

EVAPORATION RATE (BuAc = 1): NA

APPEARANCE: Clear, viscous spray

ODOR: Chlorinated solvent odor

SECTION 4					FIRE AND EXPLOSION DATA

FLASH POINT: Non-flammable

AUTOIGNITION TEMPERATURE: NA

FLAMMABILITY LIMITS IN AIR (% V): NA

EXTINGUISHING MEDIA: Foam, dry chemical, carbon dioxide

SPECIAL FIRE FIGHTING PROCEDURES: Self-contained breathing apparatus

UNUSUAL FIRE & EXPLOSION HAZARDS: Do not expose aerosols to temperatures above 130°F or the container may rupture.

SECTION 5					REACTIVITY DATA

PRODUCT STABILITY: Stable

Conditions To Avoid: Open flame, welding arcs, heat

CHEMICAL INCOMPATIBILITY: Alkalis, strong oxidizing materials, amines, potassium, sodium and magnesium

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide, hydrogen chloride, small amounts of phosgene and chlorine.

HAZARDOUS POLYMERIZATION: Will not occur

SECTION 6					HEALTH HAZARD DATA * EFFECTS OF OVER EXPOSURE

ROUTES OF ENTRY: Inhalation, Skin

HEALTH HAZARDS: Inhalation: Excessive inhalation of vapors can be harmful and may cause headache, dizziness, asphyxia, anesthetic effects and possible unconsciousness. Eyes: Irritation. Skin: Irritation due to the defatting of the skin. Ingestion: Possible chemical pneumonitis if aspirated into lungs; nausea.

CHRONIC EFFECTS OF OVER EXPOSURE: May cause elevated carboxyhemoglobin levels, vomiting and diarrhea.

MEDICAL CONDITIONS AGGRAVATED: Existing eye, skin or upper respiratory conditions

SECTION 7					EMERGENCY AND FIRST AID PROCEDURES

SKIN: Wash with soap and water. If irritation persists, seek a physician.

EYES: Flush with water for at least 15 minutes holding eyelids apart to ensure flushing of the entire eye surface. Obtain medical attention if irritated.

INGESTION: DO NOT induce vomiting. Seek medical attention immediately.

INHALATION: Move to fresh air. Resuscitate if necessary. Get medical attention.

SECTION 8					SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION: If vapor concentration exceeds TLV, use respirator approved by U.S. Bureau of Mines for organic vapor.

VENTILATION: Adequate ventilation to keep vapor concentration below TLV.

PROTECTIVE CLOTHING: If splattering is to occur, an apron is recommended.

EYE PROTECTION: Safety glasses are required.

PROTECTIVE GLOVES: Rubber gloves are required.

OTHER PRECAUTIONS: NA

WORK/HYGIENIC PRACTICES: Do not eat, drink or smoke in work area. Wash hands thoroughly before eating.

SECTION 9					ENVIRONMENTAL DATA

SPILL OR LEAK PROCEDURES: Absorb with suitable medium. Incinerate or landfill according to Federal, State or Local regulations. DO NOT FLUSH TO SEWER.

WASTE DISPOSAL METHOD: According to Federal, State and Local authorities

PRECAUTIONS IN HANDLING: Do not puncture or incinerate containers. Do not store at temperatures above 130°F. KEEP OUT OF REACH OF CHILDREN. Avoid food contamination. Avoid breathing vapors.

NA = Not Available     <= Less than     >= Greater than     ~= Approximately